R1#show inter gi 0/0

GigabitEthernet0/0 is up, line protocol is up

Hardware is iGbE, address is 0c2f.6e70.0000 (bia 0c2f.6e70.0000)

Internet address is 10.10.10.1/30

MTU 1500 bytes, BW 1000000 Kbit/sec, DLY 10 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation ARPA, loopback not set

Keepalive set (10 sec)

Auto Duplex, Auto Speed, link type is auto, media type is RJ45

output flow-control is unsupported, input flow-control is unsupported

ARP type: ARPA, ARP Timeout 04:00:00

Last input 00:00:57, output 00:00:08, output hang never

Last clearing of "show interface" counters 02:54:12

Input queue: 0/75/0/1148241 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

13994 packets input, 4986452 bytes, 0 no buffer

Received 53 broadcasts (0 IP multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored

0 watchdog, 0 multicast, 0 pause input

2363 packets output, 240353 bytes, 0 underruns

0 output errors, 0 collisions, 0 interface resets

0 unknown protocol drops

0 babbles, 0 late collision, 0 deferred

0 lost carrier, 0 no carrier, 0 pause output

0 output buffer failures, 0 output buffers swapped out

R3#show inter gi 0/0

GigabitEthernet0/0 is up, line protocol is up

Hardware is iGbE, address is 0cd2.b4d8.0000 (bia 0cd2.b4d8.0000)

Internet address is 7.7.7.1/30

MTU 1500 bytes, BW 1000000 Kbit/sec, DLY 10 usec,

reliability 255/255, txload 1/255, rxload 1/255

Encapsulation ARPA, loopback not set

Keepalive set (10 sec)

Auto Duplex, Auto Speed, link type is auto, media type is RJ45

output flow-control is unsupported, input flow-control is unsupported

ARP type: ARPA, ARP Timeout 04:00:00

Last input 00:00:57, output 00:00:05, output hang never

Last clearing of "show interface" counters 02:54:13

Input queue: 0/75/0/0 (size/max/drops/flushes); Total output drops: 0

Queueing strategy: fifo

Output queue: 0/40 (size/max)

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

53 packets input, 3180 bytes, 0 no buffer

Received 53 broadcasts (0 IP multicasts)

0 runts, 0 giants, 0 throttles

0 input errors, 0 CRC, 0 frame, 0 overrun, 0 ignored

0 watchdog, 0 multicast, 0 pause input

2356 packets output, 238948 bytes, 0 underruns

0 output errors, 0 collisions, 0 interface resets

0 unknown protocol drops

0 babbles, 0 late collision, 0 deferred

0 lost carrier, 0 no carrier, 0 pause output

0 output buffer failures, 0 output buffers swapped out

R1#show ip int br

Interface                  IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0         10.10.10.1      YES manual up                    up

GigabitEthernet0/1         unassigned      YES TFTP   administratively down down

GigabitEthernet0/2         13.1.1.1        YES manual up                    up

GigabitEthernet0/3         unassigned      YES unset  administratively down down

R1#show run | sec ospf

router ospf 1

network 0.0.0.0 255.255.255.255 area 0

R3#show ip int br

Interface                  IP-Address      OK? Method Status                Protocol

GigabitEthernet0/0         7.7.7.1         YES manual up                    up

GigabitEthernet0/1         unassigned      YES unset  administratively down down

GigabitEthernet0/2         13.1.1.2        YES manual up                    up

GigabitEthernet0/3         unassigned      YES unset  administratively down down

R3#show run | sec ospf

router ospf 1

network 0.0.0.0 255.255.255.255 area 0

Docker hub is located docker hub

Pull the trex image

[bash]>docker pull trexcisco/trex                                         #

[bash]>docker run --rm -it --privileged --cap-add=ALL trexcisco/trex      #

[bash]root@d8ec7a3a09d9 v2.36]./t-rex-64 -i                               #

[root@trexcisco-trex-2 v2.41]# cat /etc/trex_cfg.yaml

- port_limit    : 2

version       : 2

low_end       : true

interfaces    : ["veth0", "veth1"]   # list of the interfaces to bind run ./dpdk_nic_bind.py --status

port_info     :  # set eh mac addr

- ip         : 10.10.10.2

default_gw : 10.10.10.1

dest_mac   : "0c:2f:6e:70:00:00"

- ip         : 7.7.7.2

default_gw : 7.7.7.1

dest_mac   : "0c:d2:b4:d8:00:00"

[root@trexcisco-trex-2 v2.41]# cat /etc/trex_cfg.yaml

- port_limit    : 2

version       : 2

low_end       : true

interfaces    : ["veth0", "veth1"]   # list of the interfaces to bind run ./dpdk_nic_bind.py --status

port_info     :  # set eh mac addr

- ip         : 10.10.10.2

default_gw : 10.10.10.1

dest_mac   : "0c:2f:6e:70:00:00"

- ip         : 7.7.7.2

default_gw : 7.7.7.1

dest_mac   : "0c:d2:b4:d8:00:00"

[root@trexcisco-trex-2 v2.41]# ./t-rex-64 -i

Killing Scapy server... Scapy server is killed

Starting Scapy server.... Scapy server is started

The ports are bound/configured.

Starting  TRex v2.41 please wait  ...

EAL: Started without hugepages support, physical addresses not available

set driver name net_af_packet

driver capability  :

Number of ports found: 2

zmq publisher at: tcp://*:4500

wait 1 sec .

port : 0

------------

link         :  link : Link Up - speed 10000 Mbps - full-duplex

promiscuous  : 0

port : 1

------------

link         :  link : Link Up - speed 10000 Mbps - full-duplex

promiscuous  : 0

number of ports         : 2

max cores for 2 ports   : 1

max queue per port      : 3

-------------------------------

RX core uses TX queue number 0 on all ports

core, c-port, c-queue, s-port, s-queue, lat-queue

------------------------------------------

1        0      0       1       0      0

-------------------------------

-Per port stats table

ports |               0 |               1

-----------------------------------------------------------------------------------------

opackets |               0 |               0

obytes |               0 |               0

ipackets |               0 |               0

ibytes |               0 |               0

ierrors |               0 |               0

oerrors |               0 |               0

Tx Bw |       0.00  bps |       0.00  bps

-Global stats enabled

Cpu Utilization : 0.0  %  0.0 Gb/core

Platform_factor : 1.0

Total-Tx        :       0.00  bps

Total-Rx        :       0.00  bps

Total-PPS       :       0.00  pps

Total-CPS       :       0.00  cps

Expected-PPS    :       0.00  pps

Expected-CPS    :       0.00  cps

Expected-BPS    :       0.00  bps

Active-flows    :        0  Clients :        0   Socket-util : 0.0000 %

Open-flows      :        0  Servers :        0   Socket :        0 Socket/Clients :  -nan

drop-rate       :       0.00  bps

current time    : 1.8 sec

test duration   : 0.0 sec

From another terminal

muji  docker ps

CONTAINER ID   IMAGE                   COMMAND                  CREATED        STATUS       PORTS     NAMES

daf5523dd248   trexcisco/trex:latest   "/gns3/init.sh /bin/…"   12 hours ago   Up 2 hours             GNS3.trexcisco-trex-2.73354e45-bc2c-442c-a6ff-8c6138d273c2

muji  docker exec -it daf5523dd248 bash

[root@trexcisco-trex-2 v2.41]#

[root@trexcisco-trex-2 v2.41]# ./trex-console

Using 'python' as Python interpeter

Connecting to RPC server on localhost:4501                   [SUCCESS]

Connecting to publisher server on localhost:4500             [SUCCESS]

Acquiring ports [0, 1]:                                      [SUCCESS]

Server Info:

Server version:   v2.41 @ STL

Server mode:      Stateless

Server CPU:       1 x Intel(R) Xeon(R) CPU E5-2620 v4 @ 2.10GHz

Ports count:      2 x 10Gbps @ Unknown

-=TRex Console v2.0=-

Type 'help' or '?' for supported actions

trex>service -a                   # Servis moduna gir

trex>arp --port 0                 # Port 0 için ARP çözümlemesi yap

trex>service --off                # Servis modundan çık

trex(service)>ping -p 0 -d 10.10.10.1

Pinging 10.10.10.1 from port 0 with 64 bytes of data:

Reply from 10.10.10.1: bytes=64, time=23.66ms, TTL=255

Reply from 10.10.10.1: bytes=64, time=5.90ms, TTL=255

Reply from 10.10.10.1: bytes=64, time=3.80ms, TTL=255

Reply from 10.10.10.1: bytes=64, time=3.14ms, TTL=255

Reply from 10.10.10.1: bytes=64, time=4.14ms, TTL=255

trex(service)>

trex(service)>ping -p 0 -d 7.7.7.1

Pinging 7.7.7.1 from port 0 with 64 bytes of data:

Reply from 7.7.7.1: bytes=64, time=23.31ms, TTL=254

Reply from 7.7.7.1: bytes=64, time=6.44ms, TTL=254

Reply from 7.7.7.1: bytes=64, time=5.52ms, TTL=254

Reply from 7.7.7.1: bytes=64, time=4.27ms, TTL=254

Reply from 7.7.7.1: bytes=64, time=4.97ms, TTL=254

trex(service)>

trex>start -f stl/imix.py -m 10kpps --port 0

Global Statistics

connection   : localhost, Port 4501                  total_tx_L2  : 28.99 Mb/sec

version      : v2.41                                 total_tx_L1  : 30.60 Mb/sec

cpu_util.    : 0.0% @ 1 cores (1 per port)           total_rx     : 163.80 b/sec

rx_cpu_util. : 0.0% / 0.30 pkt/sec                   total_pps    : 10.02 Kpkt/sec

async_util.  : 0.16% / 1.33 KB/sec                   drop_rate    : 28.99 Mb/sec

queue_full   : 0 pkts

Port Statistics

port    |         0         |         1         |       total

----------------------------------------------------------------------

owner      |              root |              root |

link       |                UP |                UP |

state      |      TRANSMITTING |              IDLE |

speed      |           10 Gb/s |           10 Gb/s |

CPU util.  |              0.0% |              0.0% |

--         |                   |                   |

Tx bps L2  |        28.99 Mbps |          0.00 bps |        28.99 Mbps

Tx bps L1  |        30.60 Mbps |             0 bps |        30.60 Mbps

Tx pps     |        10.02 Kpps |          0.00 pps |        10.02 Kpps

Line Util. |            0.31 % |            0.00 % |

---        |                   |                   |

Rx bps     |        138.50 bps |         25.30 bps |        163.80 bps

Rx pps     |          0.26 pps |          0.03 pps |          0.30 pps

----       |                   |                   |

opackets   |            358440 |                 0 |            358440

ipackets   |                76 |                75 |               151

obytes     |         129695540 |                 0 |         129695540

ibytes     |              7929 |              7865 |             15794

opackets   |      358.44 Kpkts |            0 pkts |      358.44 Kpkts

ipackets   |           76 pkts |           75 pkts |          151 pkts

obytes     |         129.70 MB |               0 B |         129.70 MB

ibytes     |           7.93 KB |           7.87 KB |          15.79 KB

-----      |                   |                   |

oerrors    |                 0 |                 0 |                 0

ierrors    |                 0 |                 0 |                 0

status:  \

Press 'ESC' for navigation panel...

status:

trex>service -a                   # Servis moduna gir

trex>arp --port 0                 # Port 0 için ARP çözümlemesi yap

trex>service --off                # Servis modundan çık

trex>service -a

Enabling service mode on port(s) [0, 1]:                     [SUCCESS]

5.01 [ms]

trex(service)>arp --port 0

Resolving destination on port(s) [0]:                        [SUCCESS]

Port 0 - Recieved ARP reply from: 10.10.10.1, hw: 0c:2f:6e:70:00:00

608.77 [ms]

trex(service)>arp

Resolving destination on port(s) [0, 1]:                     [SUCCESS]

Port 0 - Recieved ARP reply from: 10.10.10.1, hw: 0c:2f:6e:70:00:00

Port 1 - Recieved ARP reply from: 7.7.7.1, hw: 0c:d2:b4:d8:00:00

418.57 [ms]

trex(service)>portattr

Port Status

port       |          0           |          1

-------------------------------------------------------------

driver          |    net_af_packet     |    net_af_packet

description     |       Unknown        |       Unknown

link status     |          UP          |          UP

link speed      |       10 Gb/s        |       10 Gb/s

port status     |     TRANSMITTING     |         IDLE

promiscuous     |         off          |         off

multicast       |         off          |         off

flow ctrl       |         N/A          |         N/A

--              |                      |

layer mode      |         IPv4         |         IPv4

src IPv4        |      10.10.10.2      |       7.7.7.2

src MAC         |  02:42:9f:f0:0b:00   |  02:42:9f:f0:0b:01

---             |                      |

Destination     |      10.10.10.1      |       7.7.7.1

ARP Resolution  |  0c:2f:6e:70:00:00   |  0c:d2:b4:d8:00:00

----            |                      |

VLAN            |          -           |          -

-----           |                      |

PCI Address     |         N/A          |         N/A

NUMA Node       |          -1          |          -1

RX Filter Mode  |      fetch all       |      fetch all

RX Queueing     |         off          |         off

Grat ARP        |  every 120 seconds   |  every 120 seconds

R1#show arp

Protocol  Address          Age (min)  Hardware Addr   Type   Interface

Internet  10.10.10.1              -   0c2f.6e70.0000  ARPA   GigabitEthernet0/0

Internet  10.10.10.2              1   0242.9ff0.0b00  ARPA   GigabitEthernet0/0

Internet  13.1.1.1                -   0c2f.6e70.0002  ARPA   GigabitEthernet0/2

Internet  13.1.1.2              220   0cd2.b4d8.0002  ARPA   GigabitEthernet0/2